| 1. IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND THE COMPANY/UNDERTAKING |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GHS product identifier |  |  |  |  |  |  |  |  |  |  |  |
| Product Name |  |  |  | 2001 BOOSTER FOR LAUNDRY |  |  |  |  |  |  |  |
| Other means of identification |  |  |  |  |  |  |  |  |  |  |  |
| Product Code(s) |  |  |  | 103120 |  |  |  |  |  |  |  |
| Synonyms |  |  |  | 2001 Booster |  |  |  |  |  |  |  |
| Recommended use of the chemical and restrictions on use |  |  |  |  |  |  |  |  |  |  |  |
| Recommended Use |  |  |  | Laundry Booster |  |  |  |  |  |  |  |
| Uses advised against |  |  |  | No Information Available |  |  |  |  |  |  |  |
| Supplier's details |  |  |  |  |  |  |  |  |  |  |  |
| Supplier Address Birsch Industries 476 Viking Drive Virginia Beach, VA. 23452 TEL: 757-622-0355. |
| Emergency telephone number |  |  |  |  |  |  |  |  |  |  |  |
| Emergency Telephone Number |  |  |  | CHEM-TEL, INC. 24 Hour Emergency Contact 1-800-255-3924 |  |  |  |  |  |  |  |
| 2. HAZARDS IDENTIFICATION |  |  |  |  |  |  |  |  |  |  |  |
| Classification |  |  |  |  |  |  |  |  |  |  |  |
| CORROSIVE TO METALS – Category 1 SKIN CORROSION/IRRITATION – Category 1B EYE DAMAGE/IRRITATION – Category 1 SINGLE TARGET ORGAN TOXICITY (SINGLE EXPOSURE) – Category 1 HAZARDOUS TO AQUATIC ENVIRONMENT (ACUTE HAZARD) – Category 3 |  |  |  |  |  |  |  |  |  |  |  |
| GHS Label elements, including precautionary statements |  |  |  |  |  |  |  |  |  |  |  |
| Emergency Overview |  |  |  |  |  |  |  |  |  |  |  |
| Signal Word; DANGER. |
| Hazard Statements • May be corrosive to metals • Causes serious eye damage • Causes severe skin burns and eye damage • Causes damage to organs (Respiratory System, Gastrointestinal System). |
| Appearance Clear-White.; Physical State Liquid.; Odor Characteristic.. |
| Symbol: |  |  |  |  |  |  |  |  |  |  |  |
| Precautionary Statements Prevention Do not breathe mist, vapors or spray. Wear protective gloves, protective clothing, eye and face protection. Wash thoroughly after handling. Do not eat, drink or smoke when using this product. Keep only in original container. Inhalation IF INHALED, Remove victim to fresh air and keep at rest in a position comfortable for breathing. Immediately call a POISON CENTER or doctor/physician. Ingestion IF SWALLOWED, rinse mouth. Do NOT induce vomiting. Immediately call a POISON CENTER or doctor/physician. Skin IF ON SKIN, remove immediately all contaminated clothing. Rinse skin with water/shower. Wash contaminated clothing before reuse. Eyes IF IN EYES, rinse cautiously with water for several minutes. Remove contact lenses if present and easy to do. Continue rinsing. Immediately call a POISON CENTER or doctor/physician. Other Response IF EXPOSED, call a POISON CENTER or doctor/physician. Specific treatment (See First Aid information in Section 4). Absorb spillage to prevent material damage. Storage Store locked up. Store in corrosive resistant and NON-ALUMINUM container with a resistant inner liner. (NOTE: flammable hydrogen gas may be generated if aluminum container and/or aluminum fittings are used.) Disposal Dispose of contents and container in accordance with applicable local, regional, national and/or international regulations. |  |  |  |  |  |  |  |  |  |  |  |
| 3. COMPOSITION/INFORMATION ON INGREDIENTS |  |  |  |  |  |  |  |  |  |  |  |
| Synonyms |  |  |  |  |  |  |  |  |  |  |  |
| Chemical Name |  |  |  |  |  |  | CAS-No |  | Weight % | Notes |  |
| Sodium Hydroxide |  |  |  |  |  |  | 1310-73-2 |  | < 50 | * |  |
| Sodium Chloride |  |  |  |  |  |  | 7647-14-5 |  | <35 | * |  |
| * The exact percentage (concentration) of composition has been withheld as a trade secret. |  |  |  |  |  |  |  |  |  |  |  |
| 4. FIRST AID MEASURES |  |  |  |  |  |  |  |  |  |  |  |
| Description of necessary first-aid measures |  |  |  |  |  |  |  |  |  |  |  |
| Eye Contact |  |  |  | Immediately flush contaminated eyes with a directed stream of water for as long as possible. Remove contact lenses if present and easy to do. Continue rinsing. GET MEDICAL ATTENTION IMMEDIATELY. Washing eyes within several seconds is essential to achieve maximum effectiveness. |  |  |  |  |  |  |  |
| Skin Contact |  |  |  | Immediately flush contaminated areas with water. Remove contaminated clothing, jewelry and shoes immediately. Wash contaminated areas with large amounts of water. GET MEDICAL ATTENTION IMMEDIATELY. Thoroughly clean and dry contaminated clothing before reuse. Discard contaminated leather goods. |  |  |  |  |  |  |  |
| Inhalation |  |  |  | If inhalation of mists, vapors or spray occurs and advese effects result, remove to uncontaminated area. Evaluate ABC’s (Airway constriction, Breathing, Circulation of blood) and treat symptomatically. GET MEDICAL ATTENTION IMMEDIATELY. There is no specific antidote. Treat symptomatically. |  |  |  |  |  |  |  |
| Ingestion |  |  |  | If swallowed, do NOT induce vomiting. For definite or probable ingestion, do not administer oral fluids. If vomiting occurs spontaneously, keep airway clear. Monitor airway. Volume resuscitation (IV fluids) and circulatory support (CPR) may be required. Never give anything by mouth to an unconscious or convulsive person. GET MEDICAL ATTENTION IMMEDIATELY. |  |  |  |  |  |  |  |
| Indication of immediate medical attention and special treatment needed, if necessary |  |  |  |  |  |  |  |  |  |  |  |
| Most Important Symptoms Medical Conditions Aggravated By Exposure Protection of First-Aiders Notes to Physician |  |  |  | Corrosive. This material may be corrosive to any tissue it comes in contact with. It can cause serious burns and extensive tissue destruction. May aggravate preexisting conditions such as: eye disorders that decrease tear production or have reduced integrity of the eye, skin disorders that compromise the integrity of the skin, and respiratory conditions including asthma and other breathing disorders. Protect yourself by avoiding contact with this material. Avoid contact with skin and eyes. Do not breathe vapors, sprays or mists. Do not ingest. Use personal protective equipment. Medical observation and assessment is recommended for all ingestions, all eye exposures and symptomatic inhalation and dermal exposures. For symptomatic ingestion, do not administer oral fluids and consider investigation by endoscopy, x-ray or CT scan. Esophageal perforation, airway compromise, hypotension and shock are possible. For prolonged exposures and significant exposures, consider delayed injury to exposed tissues. There is no antidote. Treatment is supportive care. Follow normal parameters for airway, breathing and circulation. Surgical intervention may be required. |  |  |  |  |  |  |  |
| 5. FIRE-FIGHTING MEASURES |  |  |  |  |  |  |  |  |  |  |  |
| Suitable Extinguishing Media Use extinguishing measures that are appropriate to local circumstances and the surrounding environment. |  |  |  |  |  |  |  |  |  |  |  |
| Specific Hazards Arising from the Chemical No determined |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous Thermal Decomposition Products Non-combustible, substance itself does not burn, but may decompose upon heating to produce corrosive and/or toxic fumes. May react with chemically reactive metals such as aluminum, zinc, magnesium, copper, etc. to release hydrogen gas which can form explosive mixtures in air. Hazardous Combustion Products Sodium hydroxide fums can be generated by thermal decomposition at elevated temeratures. Specific Fire-Fighting Methods No information available |  |  |  |  |  |  |  |  |  |  |  |
| Protective Equipment and Precautions for Firefighters Move container from fire area if it can be done without risk. Cool containers with water. Do not apply water directly on this product. Heat is generated when mixed with water. Wear NIOSH approved positive-pressure self-contained breathing apparatus operated in pressure demand mode. Avoid contact with skin. |  |  |  |  |  |  |  |  |  |  |  |
| 6. ACCIDENTAL RELEASE MEASURES |  |  |  |  |  |  |  |  |  |  |  |
| Personal precautions, protective equipment and emergency procedures |  |  |  |  |  |  |  |  |  |  |  |
| Personal Precautions |  |  |  | Do not get in eyes, on skin or on clothing. Avoid breathing mist, vapors or spray. Do not ingest. War appropriate personal protective equipment. |  |  |  |  |  |  |  |
| Environmental Precautions |  |  |  |  |  |  |  |  |  |  |  |
| Environmental Precautions |  |  |  | Keep out of water supplies and sewers. Do not flush into surface water or sanitary sewer system. This material is alkaline and may raise the pH of surface waters with low buffering capacity. Releases should be reported, if required, to appropriate agencies. |  |  |  |  |  |  |  |
| Methods and materials for containment and cleaning up |  |  |  |  |  |  |  |  |  |  |  |
| Methods for Cleaning Up |  |  |  | In cause of spill or leak, stop the leak as soon as possible, if safe to do so. Completely contain spilled materials with dikes, sandbags, etc. Shovel dry material into suitable container. Liquid material may be removed with a vacuum truck. Remaining material may be diluted with water and neutralized with dilute acid, then absorbed and collected. Flush spill area with water, if appropriate. |  |  |  |  |  |  |  |
| 7. HANDLING AND STORAGE |  |  |  |  |  |  |  |  |  |  |  |
| Precautions for safe handling |  |  |  |  |  |  |  |  |  |  |  |
| Handling |  |  |  | Avoid breathing vapor or mist. Do not get in eyes, on skin or on clothing. Wash thoroughly after handling. Do not ingest. Do not eat, drink or smoke in areas where is material is used. Wear personal protective equipment. Never add water to product. When mixing, slowly add product to water to minimize heat generation and spattering. |  |  |  |  |  |  |  |
| Conditions for safe storage, including any incompatibilities |  |  |  |  |  |  |  |  |  |  |  |
| Storage Incompatible Products |  |  |  | Store and handle in accordance with all current regulations and standards. Keep containers tightly closed and properly labeled. Do not store in aluminum containers or use aluminum fittings or transfer lines, as flammable hydrogen gas may be generated. Keep separated from incompatible substances. Acids and halogenated compounds, prolonged contact with aluminum, brass, bronzem copper, lead, tin, zinc or any other alkali sensitive metals or alloys, releases heat when diluted in water. |  |  |  |  |  |  |  |
| 8. EXPOSURE CONTROLS / PERSONAL PROTECTION |  |  |  |  |  |  |  |  |  |  |  |
| Control parameters Components with limit values that require monitoring at the workplace: Chemical Name; ACGIH TLV; OSHA PEL; NIOSH IDLH. Sodium Hydroxide 1310-73-2; Ceiling: 2 mg/m³; TWA: 2 mg/m³; -. |
| Appropriate engineering controls Provide local exhaust ventilation where dust or mists may be generated. Ensure compliance with applicable exposure limits. |  |  |  |  |  |  |  |  |  |  |  |
| Individual protection measures, such as personal protective equipment |  |  |  |  |  |  |  |  |  |  |  |
| Eye & Face Protection |  |  |  | Wear chemical safety goggles with a face-shield to protect against eye and skin contact when appropriate. |  |  |  |  |  |  |  |
| Hand & Skin Protection |  |  |  | Wear protective clothing to minimize skin contact. Wear chemical resistant clothing and rubber boots when potential for contact with the material exists. Always place pants legs over boots. Contaminated clothing should be removed, then discarded or laundered. Discard contaminated leather goods. Wear appropriate chemical resistant gloves. |  |  |  |  |  |  |  |
| Respiratory Protection |  |  |  | A NIOSH approved respirator with N95 cartridges may be permissible under certain circumstances where airborne concentrations are expected to exceed exposure limits, or when symptoms have been observed that are indicative of overexposure. If eye irritation occurs, a full face mask should be used. |  |  |  |  |  |  |  |
| Hygiene Measures |  |  |  | Handle in accordance with good industrial hygiene and safety practices. Wash hands and affected skin immediately after handling, before breaks, and at the end of the workday. When using do not eat, drink or smoke. |  |  |  |  |  |  |  |
| 9. PHYSICAL AND CHEMICAL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |
| Information on basic physical and chemical properties |  |  |  |  |  |  |  |  |  |  |  |
| Physical State; Liquid. | Appearance; Clear-White. |
| Odor; Characteristic. | Odor Threshold; No information available. |
| Property; Values. |  |  |  |
| pH; > 13. |  |  |  |
| Melting Point/Range; No data available. |  |  |  |
| Boiling Point/Boiling Range; 100 - 144° C. |  |  |  |
| Flash Point; N/A. |  |  |  |
| Evaporation rate; No data available. |  |  |  |
| Flammability (solid, gas); No data available. |  |  |  |
| Flammability Limits in Air. |  |  |  |
| upper flammability limit; No data available. |  |  |  |
| lower flammability limit; No data available. |  |  |  |
| Vapor Pressure; 13 – 135 mmHg @ 60° C. |  |  |  |
| Vapor Density; No data available. |  |  |  |
| Relative Density; No data available. |  |  |  |
| Specific Gravity; 1.53. |  |  |  |
| Water Solubility; 100. |  |  |  |
| Solubility in other solvents; No data available. |  |  |  |
| Partition coefficient: n-octanol/water; No data available. |  |  |  |
| Autoignition Temperature; No data available. |  |  |  |
| Decomposition Temperature; No data available. |  |  |  |
| Viscosity; No data available. |  |  |  |
| Flammable Properties |  |  |  | No data available |  |  |  |  |  |  |  |
| Explosive Properties |  |  |  | No data available |  |  |  |  |  |  |  |
| Oxidizing Properties |  |  |  | No data available |  |  |  |  |  |  |  |
| 10. STABILITY AND REACTIVITY |  |  |  |  |  |  |  |  |  |  |  |
| Stability |  |  |  |  |  |  |  |  |  |  |  |
| The product is stable. |
| Possibility of hazardous reactions |  |  |  |  |  |  |  |  |  |  |  |
| Mixing with water, acid or incompatible materials may cause splattering and release of large amounts of heat. Will react with some metals forming flammable hydrogen gas. Carbon monoxide gas may form upon contact with reducing sugars, food and beverae products in enclosed spaces/ |  |  |  |  |  |  |  |  |  |  |  |
| Conditions to avoid |  |  |  |  |  |  |  |  |  |  |  |
| No information available |  |  |  |  |  |  |  |  |  |  |  |
| Incompatible materials |  |  |  |  |  |  |  |  |  |  |  |
| Acids and halogenated compounds. Prolonged contact with aluminum, brass, copper, lead, tin, zinc or other alkali sensitive metals or alloys. Releases heat when diluted with water. |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous decomposition products |  |  |  |  |  |  |  |  |  |  |  |
| Toxic fumes of sodium oxide. |  |  |  |  |  |  |  |  |  |  |  |
| 11. TOXICOLOGICAL INFORMATION |  |  |  |  |  |  |  |  |  |  |  |
| Component Information Chemical Name; Oral LD50; Dermal LD50; Inhalation LC50. Sodium Hydroxide 1310-73-2; -; = 1350 mg/kg (Rabbit); -. Sodium Chloride 7647-14-5; = 3g/kg (Rat); =10 g/kg (Rabbit); 42 g/m³ (Rat) 1h. |
| Likely Routes of Exposure Skin contact, Eye contact, Inhalation, Ingestion |  |  |  |  |  |  |  |  |  |  |  |
| Symptoms |  |  |  |  |  |  |  |  |  |  |  |
| Eye Contact Skin Contact Inhalation Ingestion |  |  |  | Corrosive. Causes eye damage. Corrosive. Causes skin irritation and burns. Corrosive. May cause irritation of the respiratory tract. Corrosive. Harmful if swallowed. |  |  |  |  |  |  |  |
| Chronic Toxicity |  |  |  |  |  |  |  |  |  |  |  |
| NAM Carcinogenicity |  |  |  | No known significant effects or critical hazards. |  |  |  |  |  |  |  |
| Mutagenicity |  |  |  |  | No known significant effects or critical hazards. |  |  |  |  |  |  |
| Teratogenicity |  |  |  | No known significant effects or critical hazards. |  |  |  |  |  |  |  |
| Developmental Effects Fertility Effects |  |  |  | No known significant effects or critical hazards. No known significant effects or critical hazards. |  |  |  |  |  |  |  |
| 12. ECOLOGICAL INFORMATION |  |  |  |  |  |  |  |  |  |  |  |
| Toxicity No further relevant information available. Behavior in Environmental Systems This material is inorganic and not subject to biodegradation. This material is alkaline and may raise the pH of surface waters with low buffering capacity. This material is believed to exist in the disassociated state in the environment. Additional Ecological Information This material has exhibited slight toxicity to terrestrial organisms. This material has exhibited moderate toxicity to aquatic organisms. |  |  |  |  |  |  |  |  |  |  |  |
| 13. DISPOSAL CONSIDERATIONS |  |  |  |  |  |  |  |  |  |  |  |
| Waste Disposal Methods Uncleaned Packaging |  |  |  | Disposal should be in accordance with applicable regional, national and local laws and regulations. Disposal should be in accordance with applicable regional, national and local laws and regulations. |  |  |  |  |  |  |  |
| 14. TRANSPORT INFORMATION |  |  |  |  |  |  |  |  |  |  |  |
| DOT Not Regulated |  |  | IATA IMDG Not Regulated Not Regulated |  |  |  |  |  |  |  |  |
| 15. REGULATORY INFORMATION |  |  |  |  |  |  |  |  |  |  |  |
| International Inventories TSCA DSL |  |  |  | Complies Complies |  |  |  |  |  |  |  |
| U.S. Federal Regulations Section 313 of Tital III of the Superfund Amendments and Reauthorization Act of 1986 (SARA). This product does not contain any chemicals which are subject to the reporting requirements of the Act and Title 40 of the Code of Federal Regulations, Part 372. SARA 311/312 Hazard Categories No information available Clean Water Act This product does not contain any substances regulated as pollutants pursuant to the Clean Water Act (40 CFR 122.21 and 40 CFR 122.42). CERCLA This material, as supplied, contains substances regulated as hazardous substances under the Comprehensive Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302) or the Superfund Amendments and Reauthorization Act (SARA) (40 CFR 355). There may be specific reporting requirements at the local, regional, or state level pertaining to releases of this material: 1310-73-2 – Sodium Hydroxide California Proposition 65 This product does not contain any substance regulated by California Proposition 65. US State Right-To-Know Regulations Chemical Name; New Jersey; Pennsylvania; Massachusetts; Rhode Island. Sodium Hydroxide 1310-73-2; X; X; X; X. |
| 16. OTHER INFORMATION |  |  |  |  |  |  |  |  |  |  |  |
| NFPA. | Health 3. | Flammability 0. | Instability 2. |  |  |
| HMIS. | Health 3. | Flammability 0. | Physical Hazards 2. |  |  |
| Prepared By |  |  |  | Birsch Industries 476 Viking Drive Virginia Beach, VA. 23452 TEL: 757-622-0355 |  |  |  |  |  |  |  |
| Issuing Date |  |  |  | No information available |  |  |  |  |  |  |  |
| Revision Date |  |  |  | 19-March 2015 |  |  |  |  |  |  |  |
| Revision Note |  |  |  | N/A |  |  |  |  |  |  |  |
| General Disclaimer The information provided on this SDS is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guide for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered as a warranty or quality specification. The information relates only to the specific material designated and may not be valid for such material used in combination with any other material or in any process, unless specified in the text. |  |  |  |  |  |  |  |  |  |  |  |
| End of Safety Data Sheet |  |  |  |  |  |  |  |  |  |  |  |